Home > Products > Compressor Kits > C Model Compressors > 90 Series > 90C

VIAIR 90 Series compressors offer a light duty solution for smaller air requirements and are meant to fill no larger than a 1.0 gallon air tank. Multiple duty cycle, mounting, compressor finish and intake options are available depending on model.

Included Parts 
- Installation instructions included
- Mounting hardware included 
- Vibration isolators included 
- Check valve included Specifications 
12-Volt
Permanent Magnetic Motor
Duty Cycle: 9% @ 100 PSI
Max. Working Pressure: 120 PSI
Max. Amp Draw: 10 Amps Tech Links 
Dimensional Drawings
Recommended Wiring Diagram

| Performance Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PSI | CFM | A | BAR | LPM | A |
| 0 | 1.03 | 7 | 0 | 29.0 | 7 |
| 10 | 0.71 | 8 | 1.0 | 19.0 | 8 |
| 20 | 0.67 | 8 | 2.0 | 18.0 | 9 |
| 30 | 0.64 | 9 | 3.0 | 16.5 | 9 |
| 40 | 0.60 | 9 | 4.0 | 15.0 | 10 |
| 50 | 0.57 | 9 | 5.0 | 13.5 | 10 |
| 60 | 0.53 | 10 | 6.0 | 12.5 | 10 |
| 70 | 0.48 | 10 | 7.0 | 11.0 | 9 |
| 80 | 0.45 | 10 | 8.0 | 10.0 | 9 |
| 90 | 0.43 | 10 |  |  |  |
| 100 | 0.39 | 9 |  |  |  |
| 110 | 0.36 | 9 |  |  |  |
| 120 | 0.34 | 9 | *Supply Voltage: 13.8 Volts |  |  |

| Fill Rates |  |
| --- | --- |
| 0.5 Gallon Tank | Fill Rate |
| 0 To 80 PSI | 48 Sec. |
| 55 To 80 PSI | 16 Sec. |
| 0 To 105 PSI | 1 Min. 08 Sec. |
| 85 To 105 PSI | 14 Sec. |
| 0 to 120 PSI | 1 Min. 23 Sec. |
| 90 to 120 PSI | 25 Sec. |
| 1.0 Gallon Tank | Fill Rate |
| 0 To 80 PSI | 1 Min. 48 Sec. |
| 55 To 80 PSI | 35 Sec. |
| 0 To 105 PSI | 2 Min. 20 Sec. |
| 85 To 105 PSI | 30 Sec. |
| 0 to 120 PSI | 3 Min. 04 Sec. |
| 90 to 120 PSI | 54 Sec. |