Comparison

|  | 2000-2003 Full-Size RC | 2004-2005 Full-Size RC | 2006-2008 Full-Size RC | 2003 Edu Mini-RC | 2004-2005 Edu Mini-RC | 2006-2008 Vex Mini-RC |
| --- | --- | --- | --- | --- | --- | --- |
| Inputs and Outputs |  |  |  |  |  |  |
| Interrupt Inputs | none | 6 | <- | none | 6 | 6 |
| Digital I/O | 16 | 16, Each can be In or Out | <- | 16 | 16 User Definable as A/D In (10-bit) or Digital I/O | 16 User Definable as A/D In (10-bit) or Digital I/O |
| Analog Inputs |  | 16 (10-bit) | <- |  |  |  |
| Digital Input Sampling Frequency (typical) |  | 100KHz | <- |  | 100KHz | 100KHz |
| Analog Input Access Time |  | 10μSec | <- |  | 10μSec | 10μSec |
| Relay Outputs | 8 solenoid | 16 (8 Fwd, 8 Rev) | <- | 8 solenoid | 6 (Solenoid Drivers) | none |
| PWM Outputs, Standard refresh (17mSec) | 8 (standard) | 12 (standard) | <- | 8 (standard) | 8 total (standard or fast) | 8 standard |
| PWM Outputs, Fast refresh (up to 2mSec) |  | 4 (fast) | <- |  |  |  |
| Serial Ports | 2 (9600 Baud) | TTL Serial (115Kb) and RS232 Program (115Kb) | <- | 2 (9600 Baud) | TTL Serial (115Kb) and RS232 Program (115Kb) | TTL Serial (115Kb) and RS232 Program (115Kb) |
| User Programmable Micro-Controller |  |  |  |  |  |  |
| User Microcontroller Brand | Parallax | Microchip PICmicro® | <- | Parallax | Microchip PICmicro® | Microchip PICmicro® |
| User Microcontroller Type | Basic Stamp 2sx | PIC18F8520 | PIC18F8722 | Basic Stamp 2sx | PIC18F8520 | PIC18F8520 |
| User Processor Speed | 10KIPS (50MHz) | 10 MIPS* | <- | 10KIPS (50MHz) | 10 MIPS* | 10 MIPS* |
| Variable Space | 32 bytes + 64 bytes EEPROM | 2048 bytes + 1024 bytes EEPROM | 3936 bytes + 1024 bytes EEPROM | 32 bytes + 64 bytes EEPROM | 2048 bytes + 1024 bytes EEPROM | 2048 bytes + 1024 bytes EEPROM |
| Program Space | 16K (8 banks of 2K) + 2nd separate 16K space | 32K | 128K | 16K (8 banks of 2K) + 2nd separate 16K space | 32K | 32K |
| Programming Language | pBasic | PIC C ¦ EasyC ¦ RobotC | <- | pBasic | PIC C | PIC C ¦ EasyC ¦ RobotC |
| Programming Tools | Basic Stamp Editor | Microchip MPLAB IDE | <- | Basic Stamp Editor | Microchip MPLAB IDE | Microchip MPLAB IDE |
| General Features |  |  |  |  |  |  |
| Main Battery | 12V Robot battery | 12V Robot battery | <- | 7.2V battery | 7.2V battery | 7.2V battery |
| Battery Backup for RC and Radio | none | 7.2V Backup battery | <- | none | none | none |
| Tether to Operator Interface | none | Yes | <- | none | none | none |
| Size (W x L x H) | 6" x 7" x 0.9" | 6" x 7" x 0.9" | <- | 3.4" x 4.6" x 0.75" | 3.4" x 4.6" x 0.75" | 3.5” x 3.75” x 1” |
| Weight | 0.67 lbs. | 0.67 lbs. | <- | 0.24 lbs. | 0.24 lbs. |  |