2014 CRIO Robot: V3

As of 02/16/14

Analog Module 1:

- Tilt pot

- Winch pot

- unused

- unused

- unused

- unused

- unused

- Battery

Digital Input/Output:

- Launch pin (in)

- Launch pin (out)

- Third Leg (in)

- Third Leg (out)

- Pull up

- Pull down

- unused

- unused

- unused

- unused

- unused

- unused

- unused

- unused

Relay Outputs:

- Launch pin (Window motor)

- Third leg (Window motor)

- Camera Light

- unused

- unused

- unused

- unused

- unused

PWM:

- Left drive motors (Talon) Y-cable

- Right drive motors (Talon) Y-cable

- Winch BAG motors (Talon) Y-cable

- Winch PG motor (Talon)

- Ball Collector (Talon)

- Tilt motor (Talon)

- Winch pawl (Servo)

- unused

- unused

- unused

Solenoid Output Module

N/A

Robot user switches:

N/A

Operator Interface:

- USB 1 - Joystick 1 – left tank

- USB 2 - Joystick 2 – right tank

- USB 3 – ?

- Launch

- Winch pullback

- Ball pickup

- Tilt angle

Sequences:

Firing Sequence:

- Loosen Winch BAGS

PG opposite direction

- BAGs stop at pot setting (adjustable)

PG stops at limit switch

- Launch pin engages to limit switch

- BAGs reverse to pot setting (adjustable)

- Driver commands FIRE

Launch pin disengages to limit switch

Tilt

- Driver controlled tilt

- Pot limits to travel in both directions

- Pot setting for frame perimeter (optional)

- Pot setting for 20” beyond frame where Third Leg kicks back (to limit switch). Ahead of which Third Leg moves into shooting position.

- Driver engages Ball Collector in either direction.

Autonomous

- Camera already running

- Turn camera light on

- Decide if Hot or Not

- Tilt to shooting position

- Fire or Wait 5 seconds then Fire

- Drive forward (timed)